DEPLOYMENT & MODEL LOADING

Continuum OS: Software Deployment & LLM Loading

How the built Continuum OS software actually reaches Raspberry Pi 5 hardware, and the real, tested mechanism by which model weights are demand-paged into a running cluster.

| Doc | ARCH-004 |
| --- | --- |
| Status | Grounded in the built codebase at ~/continuum-os |
| Date | 2026-09-19 |
| Companion to | ARCH-001 (architecture), ARCH-002 (hardware/build), ARCH-003 (runtime diagram) |

# Contents

00   Scope — what this document describes

01   From source tree to a bootable node image

02   Node provisioning — from generic image to a running role

03   continuumd's own boot sequence (PID 1)

04   The weight file — how a model is packaged

05   Loading a model into the cluster, step by step

06   Tiered page resolution and what's simulated vs. real

07   Honest status — what's verified, what isn't

# 00  Scope — what this document describes

ARCH-002 specified a hardware/build plan; this document describes what actually happened when that plan was built. Every path, port number, struct field, and function name below is copied from the real source tree at ~/continuum-os, not re-derived from the architecture docs — where the build simplified, skipped, or simulated something the architecture called for, that gap is called out explicitly in §07 rather than papered over.

TWO QUESTIONS, TWO HALVES §§01–03 answer "how does the software get onto the hardware" — from a monorepo checkout to a running continuumd supervising services on a Pi 5. §§04–06 answer "how does a model get into the cluster" — from a weight file on disk to resident, dequantized tensors inside a running shard-execd process, via swapd's demand-paging mechanism.

# 01  From source tree to a bootable node image

Continuum ships as one Buildroot-based node image, assembled in two stages: this repo's own CMake build compiles every daemon, then Buildroot's post-build.sh hook stages those binaries into the target root filesystem. The image is role-agnostic — the same image boots a control-plane node or a compute node; role is decided later, in §02.

| 01 | Cross-compile every daemon first: cmake --preset aarch64-musl -S . -B build-aarch64 then cmake --build build-aarch64 --target continuumd node-agentd swapd shard-execd schedulerd api-gatewayd page-directoryd membershipd consoled |
| --- | --- |
| 02 | Point Buildroot at that output: CONTINUUM_BUILD_DIR=$(pwd)/build-aarch64 make, using image/buildroot/continuum_defconfig |
| 03 | Buildroot builds the kernel (with image/kernel/continuum-fragment.config's io_uring/userfaultfd/cgroup-v2 requirements) and the base rootfs, then invokes post-build.sh $TARGET_DIR |
| 04 | post-build.sh copies build-aarch64/kernel/continuumd to ${TARGET_DIR}/init — not a file under /opt, but literally /init, because BR2_INIT_NONE=y in the defconfig hands PID 1 straight to it |
| 05 | post-build.sh copies each of the eight service binaries into ${TARGET_DIR}/opt/continuum/bin/, and creates empty /run/continuum and /var/lib/continuum |
| 06 | post-build.sh copies the config *templates* (image/buildroot/config-templates/) into ${TARGET_DIR}/etc/continuum/ — deliberately the generic templates, not a filled-in config, so one image build is never accidentally specific to one node |
| 07 | Buildroot packages the result into a single flashable image |

Fig. 1 — image/buildroot/post-build.sh's actual staging sequence (services/*, kernel/src/main.c, image/buildroot/post-build.sh).

WHY CONTINUUMD IS `/INIT`, NOT A SYSTEMD UNIT BR2_INIT_NONE=y in continuum_defconfig tells Buildroot not to install any init system at all. The kernel's own init= boot argument points at /init directly, and that file is continuumd itself. There is no systemd, no shell-driven init, and no package manager anywhere in the image — kernel/src/main.c's own header comment calls this out: continuumd "is PID 1 on every node", the thing that makes the distributed-OS framing literal rather than aspirational at this hardware tier.

The resulting rootfs layout, as staged by post-build.sh:

/init                          continuumd (PID 1)

/opt/continuum/bin/

node-agentd  swapd  shard-execd  schedulerd

api-gatewayd  page-directoryd  membershipd  consoled

/etc/continuum/                config templates (role-specific set copied in at §02)

/run/continuum/                created empty; sockets/pidfiles land here at runtime

/var/lib/continuum/            created empty; per-node persistent state

# 02  Node provisioning — from generic image to a running role

The image from §01 is identical for every board; a node's role — control-plane or compute — and its identity (node ID, peer addresses, ports, file paths) are filled in afterward, per image/buildroot/config-templates/README.md's own instructions: copy the config set matching a node's role into /etc/continuum/, then fill in node_id, peer IPs, and paths before first boot.

## Which services a role runs

Each role's continuumd.conf names the exact children that role's continuumd supervises — read directly from the checked-in templates:

| Role | continuumd supervises | Reference IP (6-node cluster) |
| --- | --- | --- |
| control-plane (×3) | page-directoryd, membershipd, schedulerd, api-gatewayd, consoled | 10.0.0.11 / .12 / .13 |
| compute (×3) | swapd, node-agentd, shard-execd | 10.0.0.21 / .22 / .23 |

## Ports and sockets, as provisioned

Every value below is copied from the checked-in *.conf templates, not invented for this document:

| Service | Listen | Purpose |
| --- | --- | --- |
| api-gatewayd | TCP :8080 | OpenAI-compatible HTTP surface |
| schedulerd | unix:/run/continuum/schedulerd.sock | receives dispatch requests from api-gatewayd; routes by model name, e.g. route toy-model 10.0.0.21:7301 |
| page-directoryd | Raft :7101, client :7111 | 3-way Raft quorum across control-plane peers (peer 1/2/3 10.0.0.1{1,2,3} 7101) |
| membershipd | Raft :7102, client :7112 | a *separate* Raft group from page-directoryd, on purpose — a directory snapshot/compaction pause must never risk quorum membership stability |
| consoled | HTTP :7500, telemetry ingest :7401 | operator console + node telemetry sink |
| swapd | unix:/run/continuum/swapd.sock | demand-paging service; also opens a shared /dev/shm DRAM pool and page table (§05) |
| node-agentd | TCP :7201 | remote page fetch — how one compute node answers another's cache miss |
| shard-execd | TCP :7301 | prompt dispatch / token streaming — deliberately TCP, not a Unix socket, since this is the one hop that crosses physical boards (see §07, bug #5) |

CONFIG IS NEVER BAKED INTO THE IMAGE post-build.sh's own comment is explicit about this: node-specific config "is NOT baked into the image — it's provisioned per node at flash/first-boot time." That's what keeps one Buildroot output usable for all six boards in the reference cluster, and what makes re-provisioning a replacement board a config copy, not a rebuild.

# 03  continuumd's own boot sequence (PID 1)

Once a node's kernel hands off to /init, kernel/src/main.c runs the same sequence on every node regardless of role — only the config file's service list differs.

| 01 | Parse --config /etc/continuum/continuumd.conf (or -v for debug logging); exit if it fails to load |
| --- | --- |
| 02 | Open the boot-processor UART heartbeat link (cfg.uart_device) at 115200 8N1 — if the device isn't present, log a warning and continue with no hardware heartbeat rather than failing, so the same binary runs on a dev machine with nothing plugged in |
| 03 | Block SIGCHLD/SIGTERM/SIGINT and pick them up via a signalfd, rather than a signal handler |
| 04 | Arm two timerfds: a 250ms supervision tick, and (only if the UART opened) a heartbeat_interval_ms-period timer, default 2000ms |
| 05 | supervisor_init() + supervisor_start_all() — fork/exec every service named in this node's config as a separate, fault-isolated child process |
| 06 | Enter the epoll loop: on SIGCHLD, reap and let the supervisor decide whether to restart; on the 250ms tick, run supervision bookkeeping; on the heartbeat timer, write one 'H' byte to the boot-processor UART |
| 07 | On SIGTERM/SIGINT: signal every child, wait up to 3s for a clean exit, then return |

Fig. 2 — continuumd's actual startup, from kernel/src/main.c.

continuumd's own module comment is candid about one simplification worth repeating here: this reference build runs continuumd as an ordinary process rather than literally as kernel-assigned PID 1, so it can be developed and tested on an ordinary Ubuntu machine. The supervision, cgroup, and heartbeat logic is identical either way — only "mount /proc myself because I am PID 1" is skipped outside the real Buildroot image.

# 04  The weight file — how a model is packaged

ARCH-002 named a safetensors-style format as the eventual target. The actual build uses a much smaller custom format instead — lib/toymodel/include/toymodel_format.h's own comment explains why: a full safetensors reader is a lot of parsing machinery in service of proving one specific mechanism (demand-paging tensor bytes through swapd's tiered resolution), and that mechanism doesn't care what the header parser looks like — only that every tensor lands on a page boundary, which this format guarantees too.

## Layout

A toymodel_header_t (magic CTMDLv1, page size, per-tensor scale, and one {start_page, n_pages} location per tensor) occupies page 0 in its entirety — read directly off disk by whoever loads it, since it's metadata, not part of the demand-paged region. Every tensor after it starts on its own page boundary, and each tensor's start_page is the *same* absolute page index that swapd's paging engine uses for its pread()s — no translation table between "which page holds tensor X" and "fetch page N" is needed anywhere in the system.

tools/gen_toy_weights.c writes this file for the model actually used in end-to-end testing — byte-level vocabulary (256 tokens, so no separate tokenizer), one attention block, one FFN block, int8 weights with a single shared dequantization scale of 0.05. Running it prints the exact page layout it wrote:

| Tensor | Shape | Bytes | Pages | Page range |
| --- | --- | --- | --- | --- |
| header | — | 4,096 (page 0, metadata only) | 1 | 0 |
| embedding | [256, 32] | 8,192 | 2 | 1–2 |
| w_q | [32, 32] | 1,024 | 1 | 3 |
| w_k | [32, 32] | 1,024 | 1 | 4 |
| w_v | [32, 32] | 1,024 | 1 | 5 |
| w_o | [32, 32] | 1,024 | 1 | 6 |
| w_ff1 | [32, 64] | 2,048 | 1 | 7 |
| w_ff2 | [64, 32] | 2,048 | 1 | 8 |
| unembedding | [32, 256] | 8,192 | 2 | 9–10 |

Total: 11 pages × 4,096-byte page size = 45,056 bytes (44 KB) at 4-byte-per-dim/32-dim/64-ff_dim — trivially small on purpose, since proving the paging mechanism needs a model that's real, not one that's large. gen_toy_weights prints the exact tensor_region_bytes value a node's swapd.conf must be given to match, so the two never drift apart by hand-editing.

# 05  Loading a model into the cluster, step by step

This is the actual, tested path — gen_toy_weights → a compute node's swapd → shard-execd — as implemented in services/swapd/src/paging_engine.c, lib/pageclient/src/pageclient.c, and services/shard-execd/src/toy_model.c.

| 01 | An operator (or provisioning script) runs gen_toy_weights <path> [seed] on the compute node, producing a page-aligned weight file at the path swapd.conf's weight_file will name |
| --- | --- |
| 02 | That node's swapd.conf sets weight_file, tensor_region_bytes (matching gen_toy_weights' printed total), page_size 4096, and this node's node_id |
| 03 | swapd starts under continuumd, opens the weight file read-only, and mmap()s an anonymous region of tensor_region_bytes — this region, not the file itself, is what gets demand-paged |
| 04 | swapd registers that region with userfaultfd() in MISSING mode (try_setup_real_uffd); it also creates the node's shared page table and DRAM pool under /continuum-pt-<node_id> and /continuum-pool-<node_id> (POSIX shared memory, lib/pagetable) |
| 05 | shard-execd starts, and pageclient_connect() attaches to that same shared page table and pool — retrying for a few seconds if swapd hasn't created them yet |
| 06 | toy_model_load() reads the header directly from the file (page 0 — metadata, never demand-paged) to learn every tensor's {start_page, n_pages} |
| 07 | For every page of every tensor, shard-execd calls pageclient_get_page(pc, page_index). A hit in the shared page table returns a pointer straight into the DRAM pool — zero-copy, no IPC |
| 08 | On a miss, pageclient sends a WIRE_PAGE_FETCH_REQ to swapd over /run/continuum/swapd.sock — the identical message a *remote* node's swapd would send on a cross-node miss |
| 09 | swapd's paging_engine_ensure_resident() touches one byte of that page inside the uffd-registered region; the kernel blocks the access and delivers a fault event to swapd's fault thread |
| 10 | The fault thread resolves the page's content: pread() from the local weight file at page_index * page_size first; only on a short/failed local read does it try each configured remote peer's swapd over TCP with the same WIRE_PAGE_FETCH_REQ/RESP pair |
| 11 | swapd answers the kernel fault with UFFDIO_COPY, copies the page into this node's shared DRAM pool, and records it in the shared page table at WIRE_TIER_LOCAL_DRAM with a last-touched timestamp for later LRU eviction |
| 12 | shard-execd's pageclient_get_page() call returns; toy_model_load() copies the raw int8 bytes out, multiplies every element by the header's shared scale (0.05), and moves to the next tensor |
| 13 | Once all eight tensors are loaded, toy_model_load() logs the total page count fetched and returns a ready toy_model_t — shard-execd can now run toy_model_forward_step() entirely against already-resident float arrays in its own process memory |

Fig. 3 — one model load, from weight file on disk to resident float tensors inside shard-execd.

PLACEMENT IS A STATIC ROUTE, NOT AUTOMATIC SCHEDULING Which compute node loads which model is decided by a line in that node's control-plane peer's schedulerd.conf — e.g. route toy-model 10.0.0.21:7301 — not by any automatic placement logic. api-gatewayd forwards a request to schedulerd over its Unix socket; schedulerd looks the model name up in that static route table and dispatches WIRE_DISPATCH_PROMPT to the matching shard-execd over TCP. Multi-shard pipeline placement across several compute nodes is the architecture's stated direction but isn't what this build's route table expresses today — it's a single-hop route per model name.

# 06  Tiered page resolution and what's simulated vs. real

lib/wire/include/wire.h defines four resolution tiers — WIRE_TIER_LOCAL_DRAM (1), WIRE_TIER_REMOTE_DRAM (2), WIRE_TIER_LOCAL_NVME (3), WIRE_TIER_REMOTE_NVME (4) — matching ARCH-001's tiering model. The actual resolution code in paging_engine.c is more direct than four separate code paths: resolve_page_content() tries the local weight file with one pread(), and on failure tries each configured remote peer with one uniform PAGE_FETCH_REQ/RESP exchange. The module's own comment explains the collapse: from the requester's side, "remote DRAM" and "remote NVMe" both just mean "ask a specific peer for this page" — the tier distinction matters for *how a peer answers*, not for how a page is requested, so this reference build doesn't duplicate that logic four ways.

| Tier | What ARCH-001 specifies | What this build does |
| --- | --- | --- |
| Local DRAM | Page already resident in this node's DRAM pool | Exactly this — shm_pagetable_get() hit, zero-copy read (real) |
| Remote DRAM / Remote NVMe | Ask a peer node; peer answers from its own DRAM or NVMe depending on its own tier state | Folded into one path: WIRE_PAGE_FETCH_REQ to each configured peer in order, first usable WIRE_PAGE_FETCH_RESP wins (real, but tier-agnostic from the requester's side) |
| Local NVMe | Async read via io_uring | A synchronous pread() against the local weight file — the demand-paging *mechanism* (userfaultfd) is real; io_uring is named in ARCH-002 but not implemented, a swappable I/O-backend choice that doesn't touch the fault-handling path |
| userfaultfd itself | Real kernel demand paging | Real when available (UFFDIO_REGISTER succeeds) — falls back to an explicit page-touch-and-copy simulation (paging_engine_ensure_resident's non-uffd branch) when userfaultfd() is unprivileged or unavailable, so the same binary still runs correctly on a dev machine without CAP_SYS_PTRACE-adjacent permissions |

# 07  Honest status — what's verified, what isn't

Carried over directly from the project's own README, since a deployment document that omits it would be misleading:

- The full CMake build compiles cleanly and 20 ctest tests pass, including multi-process end-to-end tests for paging, Raft, and console telemetry — on native x86_64/glibc. Cross-compiling to aarch64-linux-musl (the actual Pi 5 target) is written into the CMake presets but has not been exercised in this environment.

- The Buildroot image itself has never been run — no Buildroot checkout was attempted (a full build is hours and gigabytes, unlike the firmware below). image/ holds real, complete config: defconfig, kernel fragment, post-build script, per-role templates — just never exercised end to end.

- The boot-processor firmware, by contrast, was actually built against a real downloaded ARM toolchain and Pico SDK: zero compiler warnings, a byte-correct .uf2 (verified UF2 magic numbers, ELF32/ARM header, entry point in the RP2040's XIP region). What's unverified is that firmware running on real Pico hardware — none was available to flash.

- io_uring is a synchronous pread() for now (§06) — a named, deliberate simplification, not an oversight.

- No durable Raft persistence — lib/raftnet keeps its log purely in-memory, correct for consensus but a full quorum restart loses history.

- The KV cache lives in shard-execd's own process memory, not routed through swapd's paged tensor region — lib/kvcache's prefix-trie is wired in for prefix dedup, but not for the tier-aware KV swapping ARCH-001 describes.

WHY THIS LEVEL OF CANDOR Every gap above was found by actually building and running the system — not by inspection. The Raft split-vote livelock, the blocking-timerfd hang, the jsmn double-count bug, and the Unix-socket-instead-of-TCP placement bug (this document's §02 already reflects the fix) were each caught by a specific end-to-end test failing. That's the argument for treating this document, and the diagram it accompanies, as a description of a system that has actually run — not a restatement of the plan that preceded it.

Continuum OS — ARCH-004, companion to ARCH-001 / ARCH-002 / ARCH-003